Laboratory For Atmospheric and Space Physics

LASP Engineering Division

University of Colorado

Boulder, Colorado

Interstellar Dust Experiment (IDEX) MOBI LASER TEST PROCEDURE Document No. 177902 As-Run No. _____________

| Approvers List |  |
| --- | --- |
| Prepared By | Melanie Fisher, Thermal Engineer |
| Approved By | See Agile for List of Approvers |
| Configuration Management |  |

| Rev | Change Description | By |
| --- | --- | --- |
| A | Initial Release | M. Fisher |

## Table of Contents

0.1	Table of Contents	2

0.2	Reference Documents	3

0.3	Applicable Documents	3

0.4	Acronyms/Abbreviations	3

1.0	Purpose	4

2.0	Scope	4

2.1	Requirements Verified	4

3.0	Hazards: Warnings and Cautions	4

3.1	Definitions	4

3.2	List of Warnings	5

3.2.1	Electrical Shock	5

3.2.2	Confined Space with Purge	5

3.2.3	Cryogenic Materials	5

3.2.4	Laser Radiation	6

3.3	List of Cautions	6

3.3.1	Electro-Static Discharge (ESD)	6

3.3.2	Contamination	6

3.4	Electrostatic Discharge (ESD) Control	6

3.5	Cleanliness	7

3.6	Non-Conformance Reporting	7

3.7	Emergency Contact Information	8

4.0	Equipment List	9

5.0	Test Article Configuration	9

6.0	Personnel	10

7.0	As-Run Procedures	11

8.0	Laser Test Description	12

9.0	Laser Test Steps	13

9.1	Pumpdown & High Vacuum Dwell	14

9.2	Ambient Laser Test	17

9.3	Cold Laser Test	21

9.4	Target Decontamination	25

9.5	Return To Ambient	26

10.0	Appendix A: Test Log	31

11.0	Appendix B: idx_data_acquire	32

## Reference Documents

| Document Ref | Title |
| --- | --- |
| 000057 | Anomaly Resolution and Reporting Plan |
| 105174 | LASP Health & Safety Plan |
| 105185 | Non-conformance Report Form |
| 105222 | Electrostatic Discharge (ESD) Control Procedure |
| 128805 | Hazard Warnings Library |

## Applicable Documents

| Document Ref | Title |
| --- | --- |
| 177878 | IDEX, Procedure, Thermal Vacuum Installation & Removal |
| 165809 | IDEX, Contamination Control Plan |
| 176561 | IDEX Aliveness Test Procedure |
| 176564 | IDEX, Target Decontamination Special Performance Test (SPT) Procedure |
| 177675 | IDEX Instrument Thermal Vacuum Test Cases |
| 177674 | IDEX Instrument TVAC Alarm Limits |
| 178138 | IDEX Instrument Thermal Vacuum Test Configurations |
| 176905 | IDEX, Test Rack, Startup/Shutdown Procedure |

## Acronyms/Abbreviations

| Acronym | Meaning |
| --- | --- |
| CPT | Comprehensive Performance Test |
| CSA | Charge Sensitive Amplifier |
| ESD | Electrostatic Discharge |
| GSE | Ground Support Equipment |
| GN2 | Gaseous Nitrogen |
| HV | High Voltage |
| IDEX | Interstellar Dust EXperiment |
| LASP | Laboratory for Atmospheric and Space Physics |
| LN2 | Liquid Nitrogen |
| OP | Operational |
| PPE | Personal Protective Equipment |
| QA | Quality Assurance |
| RGA | Residual Gas Analyzer |
| SPT | Special Performance Test |
| SSIM | Spacecraft Simulator |
| TCU | Thermal Control Unit |
| TOF | Time Of Flight |
| TVAC | Thermal Vacuum |

# Purpose

The purpose of this procedure is to perform laser testing of the IDEX instrument in the MOBI chamber with thermal control.

# Scope

This procedure assumes the following actions have been completed:

- Installation into the MOBI chamber and configure for laser testing per 177878 IDEX, Procedure, Thermal Vacuum Installation & Removal

This procedure performs laser testing from pumpdown to return to ambient conditions.  After completion of this procedure, 177878 will resume to remove IDEX from the chamber.

## Requirements Verified

No requirements are verified directly in this procedure.  Test data will be used verify the following:

- Target CSAs are operational over full range

- Target CSAs survival > 31 nC pulses

- Operation of Ion Detector at flight nominal gains with flight-like charge inputs

  - Identify when TOF signals are widened by impact plasma charge effects

- Functionality of the instrument over temperature

# Hazards: Warnings and Cautions

The following hazards may be present during the performance of this procedure.  Notes in the procedure will indicate at which step the hazards begin and end.

## Definitions

Caution: Notification in advance of a condition that may be of interest to personnel. The condition will not cause bodily harm but may cause damage to flight hardware, GSE or facilities.

Confined Space: A space that is large enough for an employee to enter fully and perform assigned work, is not designed for continuous occupancy by the employee, and has a limited or restricted means of entry or exit.

Danger: Notification in advance of potential serious injury or death.

Warning: Notification in advance of potential bodily harm, or damage to flight hardware, GSE, or facilities.

## List of Warnings

### Electrical Shock

WARNING – Electrical Shock

Electrical shock from inadvertent contact with conductor at high potential can cause bodily shock and/or electrical burn, as well as electrical damage to instrument-related hardware.  Seek immediate medical attention if bodily shock occurs. No PPE required. Perform safe-to-mate procedure immediately before connecting equipment to the flight instrument.

WARNING – Electrical Shock

Electrical shock from inadvertent contact with conductor at high potential can cause bodily shock and/or electrical burn, as well as electrical damage to instrument-related hardware.  Seek immediate medical attention if bodily shock occurs. No PPE required. Perform safe-to-mate procedure immediately before connecting equipment to the flight instrument.

### Confined Space with Purge

WARNING – Confined Space with Purge

Exposure of personnel to nitrogen atmosphere (displacement of oxygen) due to presence of continuous purge in a confined space creates an asphyxiation hazard, resulting in shortness of breath over short-term exposure and possible death over long-term exposure. Oxygen monitoring in surrounding workspace is required. Verify surrounding work area maintains oxygen levels between 19.5% and 23.5%. Do not enter confined space when purge is active. If shortness of breath is noted, or if the oxygen monitor visual or audio alarms go off, evacuate the immediate work area.

WARNING – Confined Space with Purge

Exposure of personnel to nitrogen atmosphere (displacement of oxygen) due to presence of continuous purge in a confined space creates an asphyxiation hazard, resulting in shortness of breath over short-term exposure and possible death over long-term exposure. Oxygen monitoring in surrounding workspace is required. Verify surrounding work area maintains oxygen levels between 19.5% and 23.5%. Do not enter confined space when purge is active. If shortness of breath is noted, or if the oxygen monitor visual or audio alarms go off, evacuate the immediate work area.

### Cryogenic Materials

WARNING – Cryogenic Materials

| A LN2 dewar is used to feed the IDEX Purge Suitcase and the IDEX Chamber shroud, getter, & LN2/bulb plate. Use caution while in proximity to the dewar valves and feed lines. Use thermally insulating gloves when handling cold surfaces. Steps where a cold hazard exists are marked with this symbol: |  |
| --- | --- |

### Laser Radiation

WARNING – Laser Radiation

IDEX laser testing is performed with a class IV laser.  Wall barriers will be erected around the laser.  All personnel within the barriers while the laser is on will have laser training.  Appropriate safety glasses shall be worn at all times while the laser is on.  Engineers performing the laser testing shall notify all personnel in the lab prior to powering the laser.  A laser warning sign shall be posted on the MOBI lab doors and the warning light shall be turned on when the laser is on.

| Steps where a laser beam hazard exists are marked with this symbol: |  |
| --- | --- |

## List of Cautions

### Electro-Static Discharge (ESD)

CAUTION –Electro-Static Discharge (ESD) Sensitivity

| ESD events can damage flight hardware. ESD events may not be detectable. Handling of flight electronic assemblies requires specific policies and procedures for eliminating the potential for equipment damage due to ESD transients. Adhere to LASP 105222 PROCEDURE, ELECTROSTATIC DISCHARGE (ESD) CONTROL. Steps where an ESD hazard exists are marked with this symbol: |  |
| --- | --- |

### Contamination

CAUTION – Contamination

The instrument contains surfaces that are extremely sensitive to particulate and molecular contamination.  Wear appropriate attire and operate per the IDEX Contamination Control Plan referenced above.

CAUTION – Contamination

The instrument contains surfaces that are extremely sensitive to particulate and molecular contamination.  Wear appropriate attire and operate per the IDEX Contamination Control Plan referenced above.

## Electrostatic Discharge (ESD) Control

Handling of flight electronic assemblies requires precautions for eliminating the potential for equipment damage due to ESD transients.  Steps requiring these precautions are marked with the ESD marking.  These requirements are covered in LASP Document Number 105222, Procedure for Electrostatic Discharge (ESD) Control.  Key requirements are listed below:

- All test engineers shall have in-date, qualified ESD training.

- All testing shall be performed at an ESD approved work station complete with a static dissipative work surface, a conductive floor, and in-date approval.

- Overhead or tabletop ionizer shall be used a safe distance from the flight electronics.

- Wrist straps shall be worn by any person within one meter of the flight electronics and shall be tested daily.

- An ESD control smock shall be worn in the lab at all times.

- The workspace shall be clear of clutter and all paper shall be kept outside a one meter diameter from the flight hardware.

- Relative humidity shall be maintained between 30% and 70%.

- Normal ESD precautions shall be taken (Not extremely sensitive).

## Cleanliness

The cleanliness of the flight hardware shall be maintained during testing.  Gloves shall be worn during all handling and the hardware shall be covered or stowed when not under test.

## Non-Conformance Reporting

All non-conformances shall be recorded in accordance with the LASP Anomaly Resolution and Reporting Plan (000057) for evaluation by project management and LASP QA.  The party discovering the non-compliance shall provide a detailed description of the issue including how the event happened if it is procedural.  The cognizant engineer shall provide analysis of the non-conformance regarding how it impacts operation and reliability.

## Emergency Contact Information

| Situation | Person | Primary Phone Number |
| --- | --- | --- |
| Medical Emergency | 911 | 911 |
| EHS Emergency | Bryan Browne | 911 / 303-868-5654 |
| Police / Security | Campus Police | 911 / 303-492-6666 |
| Facilities | Chris Rabenhorst | 303-735-8174 |
| IDEX Emergency (primary) | Scott Knappmiller | 720-254-4067 |
| IDEX Emergency (secondary) | Scott Tucker | 303-241-6643 |
| Test Emergency | Melanie Fisher | 626-826-2539 |

# Equipment List

Calibration information for calibrated equipment is recorded in procedure 177878.

- MOBI Red Binder

- Spacecraft Simulator (SSIM) and harnesses

- MOBI chamber with test article installed per 177878

  - Including GSE heaters and power supplies

- IDEX laptop & Accessories (chargers, USB cables, dock station, keyboard, mouse)

- Test Monitor laptop

- Big Sky Laser Ultra CFR Nd: YAG Laser System

- Thorlabs Photodiode PN DET10A2

# Test Article Configuration

IDEX Flight Model: ______________________

Spacecraft Simulator (SSIM) S/N: _____________________

# Personnel

Each person who will sign, initial, or contribute to this test must fill out their portion of the form below. This allows initials to be related back to the person who made them. Not all rows in this table must be filled out for test to proceed.

| Title | Printed Name | Initials | Signature |
| --- | --- | --- | --- |

# As-Run Procedures

The following table is to be filled out for all called procedures.  This is in lieu of recording the as-run numbers within the individual steps.

| Step # | Procedure Doc #, Description | Date/Time Performed | As-Run # |
| --- | --- | --- | --- |

# Laser Test Description

This test includes the following stages.  Laser testing will be performed twice, once at ambient temperature and once at cold temperature.

- Pumpdown & High Vacuum Hold

- Ambient Laser Test

- Cold Laser Test

- Target Decontamination

  - Decontamination will be performed to Design & Test temperatures

- Return to Ambient

# Laser Test Steps

Refer to following pages for the steps to perform laser testing.

| Step | Operation / Instruction | Operator, Date |
| --- | --- | --- |
| REF | Note: Steps may be performed out of order with cog-E approval, or if specified within the procedure. |  |
|  | Test Director/Thermal Engineer & QA Engineer - Verify all steps within procedure 177878 IDEX, Procedure, TVAC Installation and Removal have been completed up to the call of this procedure. - Verify all Equipment is ready for test and check the boxes in the Equipment List Section 4.0. Verify equipment calibration information is recorded in 177878 IDEX, Procedure, TVAC Installation and Removal - Fill in the Test Article Configuration information in Section 5.0. - Verify IDEX is powered OFF |  |
| REF | ## Pumpdown & High Vacuum Dwell |  |
|  | Instrument Engineer Activate the pressure interlock control switch on the SSIM front panel Set ion gauge to setpoint control Verify setpoint is 1E-5Torr |  |
|  | Instrument Engineer or Systems Engineer CAUTION! - VERIFY IDEX VOLTAGE AND CURRENT ARE ZERO PRIOR TO PUMPING THE CHAMBER. - VERIFY HV SAFE PLUG IS IN |  |
|  | Instrument Engineer Pump to Scrubbing Bubbles Pump down using the rough pump to approx. 50mtorr Use the scrubbing bubbles needle valve to add GN2 to approx. 3 torr |  |
|  | Instrument Engineer or Thermal Engineer Dwell at scrubbing bubbles Note: This dwell is for contamination risk reduction, there are no formal requirements. Adjustments to the time/temperature may be made with CC Engineer approval. Record Dwell Start Day/Time: ___________________________________ Maintain the dwell for 6-12 hours. Note: If the dwell is interrupted, it is not necessary to start over. You may resume once dwell temperature and pressure are re-established. Make a note of the duration of any interruption. Record Dwell End Day/Time: ____________________________________ |  |
|  | Instrument Engineer Close the scrubbing bubbles needle valve Begin pumping to high vacuum - Open the gate valve to the cryo pump when chamber pressure is ≤ 100mTorr - Turn on getter plate when chamber pressure is < 1e-4 Torr When chamber pressure is ≤ 1e-6 Torr, set MOBI Shroud, Platen, and LN2/Bulb plate to 20C as specified in 177675, IDEX Instrument Thermal Vacuum Test Cases for Laser Test Pumpdown to Hi Vac, getter ramp & hi vac hold Record Shroud setpoint: _________________ Record Platen setpoint: __________________ Record LN2/Bulb plate setpoint: _________________ Record cryo start day/time: ____________________________ Record high vac start day/time: _________________________ Note: High vacuum must be maintained for at least 12 hours to ensure instrument cavities are in equilibrium with tank pressure before it is safe to turn on HV |  |
|  | Instrument Engineer Dwell at high vacuum & RGA Allow to dwell for 12 hours from the high vac start time recorded in the previous step Record high vacuum start day/time: _____________________________ Record current day/time: ______________________________________ Continue the dwell until passing RGA is acquired - Move RGA file to the LASP network - Record RGA filename/location: ____________________________________________________________ Record Dwell End Day/Time: ___________________________________ |  |
|  | Instrument Engineer Enable power to the SSIM Emulator power supply by pressing the button on the interlock box after high vacuum dwell is complete. |  |
|  | Instrument Operator or systems Engineer or Instrument Engineer or EE Remove the HV Safe plug Insert the green HV Arm plug |  |
| REF | ## Ambient Laser Test |  |
|  | Instrument Operator - Verify that 12 hours has passed since the start of high vacuum (see step 70) Record high vacuum start day/time: _____________________________ Record current day/time: ______________________________________ Set instrument alarm limits to the TCYC limits Enter in CSTOL prompt: start idx_tcyc_limits Power on IDEX at nominal voltage Enter in CSTOL prompt: start idx_pwr_on nom, oper Load parameter table Enter in CSTOL prompt: start idx_load_pt_vac Note: This is the first ramp to high voltage. The slow ramp must be performed. Run slow ramp to high voltage Enter in CSTOL prompt: start idx_hvps_ramp Run noise trigger to set threshold for TOH HG data acquisition Enter in CSTOL prompt: start idx_noise_trigger nom, hvps, 2940.0V, 200.0V, 2500.0V, $vdet Record TOF HG max value:___________________ * Use these results to set the $thresh value in the idx_acquire_data script |  |
|  | Instrument Engineer Allow temperatures to stabilize to an acceptable level for laser testing. There is no specific stability criteria. This is at the discretion of IDEX Instrument Engineers. Record day/time stability achieved: ___________________________________ Save snapshots or photos of Labview and OASIS data. Save in the following file location: \\lasp-store.lasp.colorado.edu\projects\Phase_Development\IMAP-IDEX\11 Flight\_I&T\LASER\Screenshots\Ambient_Laser |  |
|  | Instrument Engineer & Instrument Operator Notify all personnel in the MOBI lab that the laser is about to be powered on. Place laser warning sign on the MOBI door and turn on the warning light. The sign should be removed and light turned off after the laser is powered off. Ensure all personnel working inside the wall barriers are wearing appropriate safety glasses prior to powering the laser. |  |
|  | Instrument Engineer & Instrument Operator Perform Laser Testing: Perform Laser Testing per IDEX Instrument Engineer instructions: - start idx_acquire_data $aid, TOF_HG, $threshold, 14400, 2940.0V, 200.0V, 2500.0V, $vdet (use step 100 to determine threshold) - Power on laser and shoot pulses on demand - Keep an eye on IDX SCIEVT_CNT for number of events collected. - Stop laser pulses - start idx_halt_acquire - start idx_transmit_data $aid (use same AID as used in step 1 and answer yes to erase aid if all data has been successfully transmitted) *loop 1-6 as necessary - Power off laser - Leave IDEX powered for transition cold. - Start idx_tvac_deconfig 1, 0 - Start idx_tvac_cfg5 nom, [aid] - Record power on time, threshold, power off time, and AID in daily test log - Note: Refer to Section 11.0 Appendix B for detailed information on how to fill in keywords for this script If test data or setup information is recorded/stored outside of this procedure, WebTCAD, or LabVIEW, record filename/location below: ________________________________________________________________ ________________________________________________________________ |  |
| REF | ## Cold Laser Test |  |
|  | Instrument Engineer or Thermal Engineer Cold Laser Test Overdrive ramp Set MOBI Shroud, Platen, LN2 Plate setpoints to the temperatures specified in 177675, IDEX Instrument Thermal Vacuum Test Cases for Laser Test Cold Laser Test Powered OFF Ramp Set Interface GSE Heaters and Door GSE Heater setpoints to the temperatures specified in 177675, IDEX Instrument Thermal Vacuum Test Cases for Laser Test Cold Laser Test Powered OFF Ramp Record day/time of ramp start: ____________________________________ Record day/time of ramp complete: ______________________________ Note: IDEX will be powered on during this ramp. |  |
|  | Instrument Engineer or Thermal Engineer and Instrument Operator Set LN2/Bulb plate to the temperature specified in 177675, IDEX Instrument Thermal Vacuum Test Cases for Laser Test Cold Laser Test Power ON & Dwell Adjust LN2/Bulb plate as required to maintain the IDEX baseplate temperature at -35C +/- 3C Record final LN2/Bulb plate setpoint: ________________________________ Allow temperatures to stabilize per IDEX Instrument Engineer instructions. Target temperatures are expected to reach approximately -35C. Record Dwell End day/time: ____________________________________ - Record DECON0TMP: _________________________________ - Record DECON1TMP: _________________________________ Save snapshots or photos of Labview and OASIS data. Save in the following file location: \\lasp-store.lasp.colorado.edu\projects\Phase_Development\IMAP-IDEX\11 Flight\_I&T\LASER\Screenshots\Cold_Laser Instrument Operator Cold Laser Test Powered ON Dwell Enter in CSTOL prompt: start idx_tvac_deconfig 5, [aid] Run noise trigger to set threshold for TOH HG data acquisition Enter in CSTOL prompt: start idx_noise_trigger nom, hvps, 2940.0V, 200.0V, 2500.0V, $vdet Record TOF HG max value:___________________ |  |
|  | Instrument Engineer & Instrument Operator Notify all personnel in the MOBI lab that the laser is about to be powered on. Place laser warning sign on the MOBI door and turn on the warning light. The sign should be removed and light turned off after the laser is powered off. Ensure all personnel working inside the wall barriers are wearing appropriate safety glasses prior to powering the laser. Perform Laser Testing Perform Laser Testing per IDEX Instrument Engineer instructions - start idx_acquire_data $aid, TOF_HG, $threshold, 14400, 2940.0V, 200.0V, 2500.0V, $vdet (use step 150 to determine threshold) - Power on laser and shoot pulses on demand - Keep an eye on IDX SCIEVT_CNT for number of events collected. - Stop laser pulses - start idx_halt_acquire - start idx_transmit_data $aid (use same AID as used in step 1 and answer yes to erase aid if all data has been successfully transmitted) *loop 1-6 as necessary - Power off laser - Power off IDEX: start idx_pwr_off - Record power on time, threshold, power off time, and AID in daily test log - Note: Refer to Section 11.0 Appendix B for detailed information on how to fill in keywords for this script If test data or setup information is recorded/stored outside of this procedure, WebTCAD, or LabVIEW, record filename/location below: __________________________________________________________________ __________________________________________________________________ |  |
| REF | ## Target Decontamination |  |
|  | Instrument Operator Perform 176564 IDEX, Target Decontamination Special Performance Test (SPT) Procedure - Note: Decon Heater voltage shall be nominal (31V) - Note: Ramp 5 shall be performed - Note: Record any adjustments to heater setpoints in the space below - Note: IDEX shall be powered OFF at the end of this procedure. At the discretion of the thermal engineer, IDEX may be powered OFF prior to the target cooling to ≤ 40C Inform Instrument Engineer or Thermal Engineer when Ramp 2 is completed. The Return to Ambient GSE transition may be started after the completion of Ramp 2 Inform Instrument Engineer when the 4 hour dwell after Ramp 5 begins. The RGA in the next step may be performed at that time. Power off IDEX when target temperatures cool to ≤ 40C (may be powered OFF prior with thermal engineer approval) Note: To power off prior to temperatures reaching ≤ 40C, Type GOTO POWER_OFF in Target Decontamination script Record As-Run information in the table in Section 7.0. |  |
|  | Instrument Engineer Decon RGA Note: This step may be performed any time during the 4 hour decontamination dwell Note: This is for information only, result does not impact other procedure steps Perform RGA while the target is at decontamination temperatures - Move RGA file to the LASP network - Record RGA filename/location: ____________________________________________________________ |  |
| REF | ## Return To Ambient |  |
|  | Instrument Engineer or Thermal Engineer Return to Ambient Temperature Transition Note: This step may be performed any time after the completion of Target Decontamination Ramp 2 Set MOBI Shroud, Platen, LN Plate setpoints to the temperatures specified in 177675, IDEX Instrument Thermal Vacuum Test Cases for Laser Test Return to Ambient Set Interface GSE Heaters and Door GSE Heater setpoints to the temperatures specified in 177675, IDEX Instrument Thermal Vacuum Test Cases Laser Test Return to Ambient Note: Maintain instrument temperatures above the shroud temperatures during return to ambient |  |
|  | Instrument Engineer - Verify that instrument and GSE temperatures are sufficiently warm to allow backfill to begin (Do not begin backfill in this step) Record day/time of temperature transition complete: _______________________ |  |
|  | Instrument Operator - Verify that the instrument is OFF - Verify survival heater is disabled If survival heater is enabled, perform the following: Enter in CSTOL prompt: start idx_surv_pwr_off |  |
|  | Quality Assurance Engineer - Verify all steps within this procedure up to this step have been completed and signed. Verify steps have been appropriately filled in and requested attachments are included where specified. - Verify that the As-Run table in Section 7.0 has been filled out completely |  |
|  | Test Director (Thermal Engineer) Conduct a post-test meeting to verify IDEX is ready to backfill with the following people: - Test Director/Thermal Engineer - Electrical Lead - Mechanical Lead - Systems Lead - Calibration Lead - Operations Lead - Quality Engineer - Verify all are in agreement that the test is complete and it is ok to proceed with SSIM shutdown and chamber backfill Inform all team members that 24 hour test monitoring is over. |  |
|  | Systems Engineer & Instrument Engineer & Instrument Operator - Verify all test telemetry has been uploaded to WebTCAD - Copy all data to the network server and record the file locations: ___________________________________________________________ ___________________________________________________________ ___________________________________________________________ - Perform 176905 IDEX, TEST RACK, STARTUP/SHUTDOWN PROCEDURES to power off IDEX SSIM - Remove the green HV Arm Plug and Install the HV Safe Plug |  |
|  | Instrument Engineer - Copy all LabVIEW data to the network server and list location here: ___________________________________________________________ _______________________________________________________ - Turn off LabVIEW Thermal Record, Getter Control, Limit Checking, Notifications - Turn off LN2 getter plate - Backfill with dry GN2 to partial pressure of 100-200 Torr. - Wait until getter temperature is above dew point - If planning to open the MOBI door immediately, finish backfill to atmospheric pressure and notify Mechanical Engineer, Mechanical Assembler, and QA Engineer that the door reset activity will be begin shortly. Otherwise, hold at ½ atm |  |
| REF | PAUSE TEST PROCEDURE UNTIL NORMAL WORKING HOURS |  |
|  | Instrument Engineer Complete backfill to ambient pressure if not already complete. Note: Do not open chamber door until mechanical team is ready to reset the door and put IDEX back on purge |  |
|  | QA Engineer Perform final document review. |  |
|  | END OF PROCEDURE Resume procedure 177878 IDEX, PROCEDURE, THERMAL VACUUM INSTALLATION & REMOVAL |  |

# Appendix A: Test Log

| Date: |  |  |
| --- | --- | --- |
| Instrument Operator: |  |  |
| Laser Operator: |  |  |
| Instrument Engineer: |  |  |
| Other Personnel: |  |  |
| TOF HG Trigger Threshold [DN] |  |  |
| Detector HV used [V] |  |  |
| AID Used |  |  |
| Time Power On/Off |  |  |
| Test Matrix Completed today: (rows) |  |  |
| Notes: | Signoff:. |

# Appendix B: idx_data_acquire

Instrument Operator: Change HV

- Refer to documentation on confluence for detailed notes about the keywords for this command:

https://confluence.lasp.colorado.edu/display/IMAPIDEX/2023-07-24+Pre+Environmental+Dust

- To change voltage or trigger threshold, start a new data acquisition with desired voltages/threshold using:

idx_acquire_data $aid, $channel, $threshold, $duration, $vsen, $vrjctn, $vtar, $vdet

- where

  - $aid = x#8FFDOYHH

    - DOY = Day of Year out of 365 days

    - HH = UTC Time

    - Eg: x#8FF18720 if 07/06/2023 = 187 and 1 PM MST = 20 UTC

  - $channel = specifies which TOF channel to trigger on (options: TOF_HG, TOF_MG, TOF_LG)

  - $threshold = trigger threshold (V)

  - $duration = duration of data acquire mode (s)

  - $vsen = voltage setpoint for sensor (parabolic) (V)

  - $vrjctn = voltage setpoint for rejection (V)

  - $vtar = voltage setpoint for target (V)

  - $vdet = voltage setpoint for detector (V)